Trajectory optimization

The excitation trajectory for each joint is a finite sum of N harmonic sine and cosine functions. The joint position qi(t), velocity qi(t), and acceleration qi(t) trajectories for joint i of an n-degrees-of-freedom robot are given as

qit=l=1Nailωflsinωflt-bilωflcosωflt+ci

qit=l=1Nailcosωflt+bilsinωflt

qit=l=1N-ailωflsinωflt+bilωflcosωflt

With ωf the fundamental pulsation of the Fourier series. This Fourier series specifies a periodic function with period Tf=2πωf. The fundamental pulsation is common for all joints, in order to preserve the periodicity of the overall robot excitation. Each Fourier series contains 2×N+1 parameters, that constitute the degrees of freedom for the optimization problem: ail and bil, for  l=1 to N, which are the amplitudes of the cosine and sine functions, and ci which is the offset on the position trajectory. The offset determines the robot configuration around which excitation will occur. The parameters for all joints are grouped into vector x.

For the i-th sampling point,

ui=Yiqi,qi,qiβ+ϵi

In matrix form,

u1u2⋮um=Y1(q1,q1,q1)Y1(q2,q2,q2)⋮Y1(qm,qm,qm)β+ϵ1ϵ2⋮ϵm

That is,

U=Φa,b,cβ+ϵ

The problem of determining an excitation trajectory q*(t) can be formulated as

mina,b,cκ(Φ(a,b,c))

Subject to

qmin≤qt≤qmax

q≤qmax

q≤qmax

qt0=qtf=qinit

qt0=qtf=0

qt0=qtf=0

The constraints for each joint are

qit=l=1Nailωflsinωflt-bilωflcosωflt+ci≤l=1N1ωflail2+bil2+ci≤qmax

Similarly,

qit=l=1Nailcosωflt+bilsinωflt≤l=1Nail2+bil2≤vmax

qit=l=1N-ailωflsinωflt+bilωflcosωflt≤l=1Nωflail2+bil2≤amax

Unexpected behavior will result at the start and end points if qit0≠0, qit0≠0, qitf≠0 or  qitf≠0. Therefore, constraints are added to remedy this defect.

qit0=qi0=-l=1Nbilωfl+ci=0

qit0=qi0=l=1Nail=0

qit0=qi0=l=1Nωflbil=0

Racer3-0.63减速比如下：

1轴：1:50

2轴：1:80

3轴：1:80

4轴：1:50

5轴：1:50

6轴：1:50

Franka

Limits in the Cartesian space are as follows:

Name	Translation	Rotation	Elbow

p˙max	1.7000 ms	2.5000 rads	2.1750 rads

p¨max	13.0000 ms2	25.0000 rads2	10.0000 rads2

p...max	6500.0000 ms3	12500.0000 rads3	5000.0000 rads3

Joint space limits are:

Name	Joint 1	Joint 2	Joint 3	Joint 4	Joint 5	Joint 6	Joint 7	Unit

qmax	2.8973	1.7628	2.8973	-0.0698	2.8973	3.7525	2.8973	rad

qmin	-2.8973	-1.7628	-2.8973	-3.0718	-2.8973	-0.0175	-2.8973	rad

q˙max	2.1750	2.1750	2.1750	2.1750	2.6100	2.6100	2.6100	rads

q¨max	15	7.5	10	12.5	15	20	20	rads2

q...max	7500	3750	5000	6250	7500	10000	10000	rads3

τjmax	87	87	87	87	12	12	12	Nm

τj˙max	1000	1000	1000	1000	1000	1000	1000	Nms

0.0504619686813309

-0.0843743621491376

0.100054699873042

-0.116542590051972

0.104882969123647

-0.00250849250969032

0.0441680324096574

0.0268343723158677

0.210774128780459

-0.212022437734765

0.288635561279540

-0.113346079883937

-0.0617250526613452

-0.0735193353863894

-0.0867090884393223

-0.0272512592740232

-0.0318806099118051

-0.0673452918962738

0.0490707035480160

0.0256715314318477

0.0425874415683531

0.107437626668884

-0.0562718886820616

0.0780864280694126

-0.0516471490779072

0.0539064571674477

0.0636771194520985

-0.0723485992965320

-0.0980248792267598

-0.0428766186752368

0.0657619197041153

-0.0531005302533871

-0.0945140499551737

-0.0251151057129107

0.0591124607049109

-0.117901004761665

0.0820386737829796

-0.0408750109404982

0.133255944200198

0.0579412317701598

0.0290210852539886

-0.0563100940179577

0.225384056694958

0.0687101721874361

0.179015851659392

-0.0667803718148779

0.0311995660123151

-0.504704865271416

0.391608182184045

-0.192464483547245

-0.239204293964038

-0.104611105459267

-0.0838413010560911

-0.125780221554282

0.586736041839766

-0.383343057231312

-0.0834069333278452

0.0344229338887735

0.0968358943205884

-0.0777142741760652

-0.0504790156597005

-0.0674349039815375

0.0441745255461798

0.115630815064973

0.0720924256358334

-0.0781333906565675

0.112537159860418

0.0917832727013717

-0.102015972860861

0.0492849598318169

-0.106776055937064

0.0949887849928292

0.0355629203329022

-1.38063773754516

0.0594462411301042

1.39652607052393

0.0519061586686018